OUTLINE:	Obtaining Useful Answers from Groundwater Models

NWRA	8:00 am - 12:00 pm,	Tuesday, February 3, 2026

01_GWmodels_INTRO+Basics+Real.v2.pptx

Groundwater models developed to address questions

Conceptual model should be developed before groundwater model

Groundwater models are only as good as supporting data

Water levels and discharges only direct measurements of groundwater system

02_GWmodels_Flow+Calibration.v2.pptx

Flow models provide consistent stories that can be compared to data

Lateral and vertical extents should be enough to solve problem without going too far

Hydraulic properties, stresses, and boundaries must be defined for entire model

Parameter estimation formalizes model calibration, where measured and simulated quantities are compared in an objective function

Parameters are changed so that objective function is reduced

Parameters typically define arrays of hydraulic properties with zones or pilot points

Parameter-to-array example and tube-model workbooks in SupportingMaterial\02_Interpolate+TubeMODEL subfolder

03_HydProp+GeoFrameworks.v2.pptx

Geologic frameworks distribute hydraulic properties through groundwater-flow models

Groundwater controlled by hydraulically connected fractures

Detailed, complex frameworks do not improve hydraulic-property estimates

Compare transmissivity, not hydraulic conductivity, to rock types

04_Looking+CheckingShapes.v2.pptx

Calibration should be evaluated with more than statistics, such as RMS error

Visual comparison of model results required to evaluate model plausibility

Maps and plots provide better visualization, compared to tables

Looking necessary to cross bar of “Not obviously wrong”

Broad visualization of simulated results uncovers undefined badness, 
which leads to better models

Simplifying model features and integrating results for mapping eases comprehension

Cheap tools such as Model Viewer and Google Earth encourage looking by more users

Transmissivity distributions simplify presenting model results and aids comprehension

Water-level profiles are processed quickly and function 
like hydrographs for steady state models

05_GeologicFrameworks_Dos+Donts.v2.pptx

Geologic frameworks are not data, Distinguish between interpretation and reality

Geologic frameworks are foundations for distributing hydraulic properties

Revise geologic framework based on hydrologic data

Truncate geologic frameworks based on groundwater model purpose

Detailed, complex frameworks are not always better than simplified frameworks

06_Uncertainy.v2.pptx

Calibration & uncertainty analysis depend on judgement

Model developers’ judgement explicitly embedded in objective function and parameters

Objective function—Data exclusion, data inclusion, and weighting of included data

Parameters—Spatial extent of faults or zones and pilot-point density, 
	Ranges of defined parameters, where textbook ranges allow too much

Uncertainty analysis is not free and can cost measuring useful data

07_Apply+Report.v2.pptx

Results not overly sensitive to detail of model grid

Increasing discretization increases model run time, which weakens calibration and
post-processing of results

Models flexible—Answer many questions and are improved by clear purpose

Data and questions should drive model complexity

Additional data dispels ignorance, not more models

Show added details improve answers

Clear ties to measurements increase confidence

A figure of integrated model results is easier to comprehend than multiple figures and tables that need to be integrated mentally.

Require complete reporting

Interior of Earth can be mysterious,

No aspect of a model should be mysterious

Workbook 01_DD_CoarseFineGRID_Compare.xlsm and supporting files for creating, executing, and viewing coarse and fine models in SupportingMaterial\07_Apply+Grid+Report subfolder